Asynchronous FIFO Design Specification

1. Introduction

1.1 Module Name

asynchronous_fifo

1.2 Description

The asynchronous_fifo module implements an Asynchronous First-In-First-Out (FIFO) memory that enables reliable data transfer between two independent clock domains. The design employs Gray-coded read and write pointers, two-stage synchronizers, and dual-port memory to safely handle clock domain crossing (CDC).

The FIFO supports:

- Independent write and read clocks.

- Independent asynchronous resets.

- Full and empty status indication.

- Configurable FIFO depth and data width.

2. Features

- Asynchronous clock domain operation.

- Parameterized FIFO depth.

- Parameterized data width.

- Binary and Gray-coded pointer implementation.

- Two-stage synchronizers for clock-domain crossing.

- Full detection.

- Empty detection.

- Dual-port memory architecture.

- Non-blocking write and read operations.

Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| DEPTH | 8 | FIFO depth |
| DATA_WIDTH | 8 | Width of each FIFO data word |

Derived parameter

PTR_WIDTH = clog2(DEPTH)

Inputs

| Signal | Width | Description |
| --- | --- | --- |
| wclk | 1 | Write clock |
| wrst_n | 1 | Active-low write reset |
| rclk | 1 | Read clock |
| rrst_n | 1 | Active-low read reset |
| w_en | 1 | Write enable |
| r_en | 1 | Read enable |
| data_in | DATA_WIDTH | Input data |

Outputs

| Signal | Width | Description |
| --- | --- | --- |
| data_out | DATA_WIDTH | Output data |
| full | 1 | FIFO full indication |
| empty | 1 | FIFO empty indication |

6. Internal Architecture

The FIFO consists of five major blocks.

6.1 Synchronizer

Purpose

Transfers Gray-coded pointers safely across asynchronous clock domains.

Instances

Write Pointer Synchronizer

Transfers Gray write pointer into read clock domain.

g_wptr

↓

Synchronizer

↓

g_wptr_sync

Clock domain:

Write → Read

Read Pointer Synchronizer

Transfers Gray read pointer into write clock domain.

g_rptr

↓

Synchronizer

↓

g_rptr_sync

Clock domain:

Read → Write

Functional Description

Two flip-flops are connected in series.

d_in

│

▼

FF1

│

▼

FF2

│

▼

d_out

This significantly reduces metastability caused by asynchronous clock crossing.

7. Write Pointer Handler

Purpose

Maintains the write pointer and generates the Full flag.

Inputs

| Signal | Description |
| --- | --- |
| wclk | Write clock |
| wrst_n | Reset |
| w_en | Write enable |
| g_rptr_sync | Synchronized Gray read pointer |

Outputs

| Signal | Description |
| --- | --- |
| b_wptr | Binary write pointer |
| g_wptr | Gray write pointer |
| full | FIFO full |

Operation

Binary Pointer

b_wptr_next =

b_wptr + (w_en & !full)

Gray Conversion

Gray =

Binary XOR (Binary >> 1)

Full Detection

The FIFO is full when

g_wptr_next ==

{

~g_rptr_sync[MSB],

~g_rptr_sync[MSB-1],

g_rptr_sync[Remaining Bits]

}

This indicates the write pointer has wrapped around and caught the read pointer.

8. Read Pointer Handler

Purpose

Maintains the read pointer and generates the Empty flag.

Inputs

| Signal | Description |
| --- | --- |
| rclk | Read clock |
| rrst_n | Reset |
| r_en | Read enable |
| g_wptr_sync | Synchronized Gray write pointer |

Outputs

| Signal | Description |
| --- | --- |
| b_rptr | Binary read pointer |
| g_rptr | Gray read pointer |
| empty | FIFO empty |

Operation

Binary Pointer